dGray’s Furniture Restoration & Refinishing

Production Overview

# Pick up

Repair (when dismantling)

-Loose joints

-Loose veneer

-Loose dovetail joints

## Refurbishing

-Repair and/or fabricate missing parts, splinters, and broken or cracked areas

## Stripping

-Hand strip in stainless steel pan outdoors

-Continuously pour stripper over select area until finish is removed

Neutralize: with lacquer thinner and air-dry one day

Sand: with progressively finer sand paper

Sealer: spray of pre lacquer

Fill pores: with a wood pore filler of appropriate color

## Sand

## Finish: minimum of 3 coats of lacquer-sanding in between

-Background color tone lacquer

-Penetrating tinted lacquer for blending of colors of wood/grain

-Detail/ touch-up

-Blending coat of lacquer

## Final clear coat to finish

Reassemble polished hardware

Deliver

Please visit www.dgrays.com for a detailed process:

>Step-by-Step Detailed

>Step-by-Step Pictorial